Activity Hazard Analysis (AHA) CHECKLIST	Project Name:	Task:

|  | Hazards | Controls | PPE | Permits |
| --- | --- | --- | --- | --- |
| [ ] | Manual Lifting | [ ] Warm up [ ] Lift object: close to body, avoid twisting, use legs, face the load [ ] < 40 lb./person [ ] Team Lift | [ ] Select |  |
| [ ] | Overhead Utility Pole Wire | [ ] Vehicles > 20 ft. (truck, aerial/scissor lift, crane) from overhead electric lines | [ ] Select | [ ] EEWP |
| [ ] | Electric Power | [ ] Barricade [ ] Determine all outputs [ ] LTT [ ] 2-person rule if > 50V [ ] Verify de-energized with AC voltage detector [ ] Re-energization plan in place [ ] Shock boundaries [ ] Attendants | [ ] Select | [ ] EEWP |
| [ ] | Hoist or Overhead Crane | [ ] Signal person [ ] Tag lines [ ] Barricaded [ ] Crane & connections inspected | [ ] Select |  |
| [ ] | Mobile Crane | [ ] Signal person [ ] Tag lines [ ] Barricaded [ ] Crane & connections inspected | [ ] Select | [ ] LIFT |
| [ ] | Forklift | [ ] Pre-use inspected [ ] Safe speed [ ] Secure load [ ] Spotter [ ] Use horn [ ] Reverse driving if front view blocked [ ] Seatbelt [ ] On grades, tilt load back | [ ] Select | [ ] LIFT (When using attachments) |
| [ ] | Trench/Dig/ Excavate | [ ] Utilities & Safe dig zone identified [ ] If > 5 ft. depth: shore, slope, or shield [ ] If > 4 ft. depth: Egress every 25 ft., Ventilate & 4-Gas Testing | [ ] Select | [ ] DIG |
| [ ] | Floor/Wall/Ceiling | [ ] Utilities & Safe penetration zone identified in Floor, Wall, or Ceiling [ ] Drill stops |  | [ ] PENETRATION |
| [ ] | Weld/Cut/Braze/Grind | [ ] Fire Watch with fire extinguisher [ ] Flammables & combustibles removed [ ] Flammable & combustibles covered [ ] Welding screens [ ] GFCI [ ] Ventilation | [ ] Select | [ ] HW |
| [ ] | Vehicle Traffic | [ ] Cones [ ] Barricaded & Signs [ ] Traffic Flagman [ ] Signalman [ ] High-Vis Vest | [ ] Select |  |
| [ ] | Hand & Power Tool, Cord | [ ] GFCI [ ] Pre-use inspected [ ] Guarded [ ] No electric tools in wet locations | [ ] Select |  |
| [ ] | Ladder (portable & fixed) | [ ] Pre-use inspected [ ] Climb no tools in hands [ ] Rated Type I [ ] No over reach [ ] 4:1 angle [ ] Ladder tied off/held [ ] 3 points of contact [ ] No stand top steps | [ ] Select |  |
| [ ] | Scaffold | [ ] Pre-use inspected [ ] Tags current [ ] Footings solid [ ] Secured to structure [ ] Guardrails & toe-boards used [ ] Ladder installed & secured [ ] Gates | [ ] Select |  |
| [ ] | Overhead Object Dropped | [ ] Toe-board, screen, debris net [ ] Tools tethered [ ] Scrap chutes [ ] Barricade | [ ] Select |  |
| [ ] | Scissor Lift & Aerial/MEWP | [ ] Pre-use inspect [ ] Firm surface [ ] Work inside guardrail [ ] Spotter [ ] Barricade | [ ] Select |  |
| [ ] | Slip, Trip or Fall | [ ] Housekeeping [ ] Hoses/cords organized [ ] Spills prevented [ ] Hole cover | [ ] Select |  |
| [ ] | Line-Of-Fire | [ ] Tool guards [ ] Line-of-Fire clearance area [ ] Barricades | [ ] Select |  |
| [ ] | Compressed Gas Cylinders | [ ] Pre-use inspected [ ] Secured & upright [ ] Oil & grease-free cylinders [ ] Oxygen & fuel cylinders secured on welding cart or separated by 20 ft. or fire-rated wall | [ ] Select |  |
| [ ] | Very Hot or Cold Weather | [ ] Water breaks [ ] Rest breaks [ ] Shade [ ] Worker rotation [ ] Layered clothing | [ ] Select |  |
| [ ] | Severe Weather | [ ] Frequent weather forecast checks [ ] Stop work if high winds, storms, lightning | [ ] Select |  |
| [ ] | Environmental (air/waste/water) | [ ] Spill prevention [ ] Sanitary drains covered [ ] Stormwater runoff barriers [ ] Hazardous waste containers coordinated with Jlab Environmental | [ ] Select |  |
| [ ] | Adjacent Work Hazards | [ ] Barricades [ ] Identify workers adjacent, above or below [ ] Coordinated with adjacent supervisor, foreman, customer, operator | [ ] Select |  |
| [ ] | Pinch Points | [ ] Guards [ ] Hands/face/body/feet kept clear of line-of-fire [ ] Cover sharp edges | [ ] Select |  |
| [ ] | Entering Barricaded Area | [ ] Barricade ( SIGN explaining purpose of barricade required for all barricades) | [ ] Select |  |
| [ ] | Noisy Portable Equipment | [ ] If >85 decibels, “Hearing PPE Required” signs posted [ ] Dual PPE if > 100 dBA | [ ] Select |  |
| [ ] | Chemical Use (IH approval) | [ ] Reviewed SDS [ ] Exposure air monitoring [ ] Containers labeled [ ]Ventilation [ ] Containers large enough & compatible to chemical [ ] Spill containment | [ ] Select | [ ] PRCS |
| [ ] | Silica Dust (IH approval) | [ ] Cut/drill/grind concrete & brick manually without power tools [ ] Wet methods [ ] Power tools use HEPA vacuum or water spray [ ] Barricades [ ] IH approval | [ ] Select |  |
| [ ] | Oxygen Deficiency Hazard (ODH Areas 0/1/2/3/4) | [ ] Prevent escape of compressed gasses or fire suppression chemicals [ ] Buddy system for ODH 1,2,3, or 4 areas [ ] Respirator for ODH 2,3, or 4 areas | [ ] Select |  |
| [ ] | Cryogenic | [ ] Identify work with helium, nitrogen, argon, hydrogen, oxygen gasses | [ ] Select |  |
| [ ] | Asbestos, Lead, Beryllium | [ ] Industrial Hygiene approval if disturbing (cut, drill, grind, remove) [ ] VA License | [ ] Select |  |
| [ ] | Non-Ionizing Radiation | [ ] Industrial Hygiene approval for work with either lasers, magnetic fields, or RF | [ ] Select |  |
| [ ] | Drill/Cut in (RCA) Ionizing Radiation Controlled Areas | [ ] RadCon approval [ ] Dosimeter [ ] SRPD Dosimeter [ ] Barricades | [ ] Select | [ ] RWP |
| [ ] | Respirator Use | [ ] Industrial Hygiene approval of work plan | [ ] Select |  |
| [ ] | Body Harness / Lanyard Use | [ ] TR approval of subcontractor work plan [ ] Supervisor approval of FML work plan | [ ] Select | [ ] FALL |

PERMITS

PRCS (Permit-Required Confined Space) / LIFT (Critical Lift) / HW (Hot Work Non-Electrical) / LASER (Laser) / PENETRATION

EEWP (Energized Electrical) / FALL (Fall Protection) / RWP (Radiation Work) / DIG (Digging, Excavation, Trenching) / ASBESTOS / LEAD

PPE to Select (* = Always Required)

| General PPE [ ] *Hard Hat [ ] Hearing Protection [ ] Double Hearing Protection [ ] Hi-vis Vest, Jacket or Shirt [ ] *Long Pants | Eyes & Face [ ] *Safety Glasses [ ] Face Shield [ ] Chemical/Dust/Burning Goggles [ ] Welding Hood | Hands [ ] Cut Resistant Gloves [ ] Welders Leather Gloves [ ] Chemical Compatible Gloves [ ] Surgical Gloves [ ] Arm Sleeves | Fall Protection [ ] Full Body Harness [ ] Lanyard or Self-Retracting Lanyard [ ] Anchorage Connector [ ] Horizontal Life Line System [ ] Stanchions & Flags |
| --- | --- | --- | --- |
| Respiratory [ ] Dust Mask [ ] Air Purifying (½, full, PAPR) [ ] Air Supplied (SCBA or Airline) [ ] Emergency Escape | Feet [ ] *Safety Toe Shoes [ ] Rubber Boots [ ] Metatarsal Protection | Special Clothing [ ] Tyvek Coveralls [ ] Fire Retardant Clothing [ ] Chemical Apron | Arc Flash [ ] Category 1 [ ] Category 2 [ ] Category 3 [ ] Category 4 |

Form Date 08/16/22

| Tracking/Submittal Number | Project | Company Name |  |
| --- | --- | --- | --- |
| Task Description |  | Competent Person / SSHR Assigned |  |
|  |  | Phone: |  |

| Created By | Date | TR/CM Approval | Date |
| --- | --- | --- | --- |

| Required Permits and Plans |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ☐ | Hot Work | ☐ | Confined Space | ☐ | Dig/Blind Penetration | ☐ | Lift Plan | ☐ | Outage |

| Required Jlab Training | Alternative Competent Person / SSHR |
| --- | --- |
| All workers: SAF100C or SAF100S |  |
| SSHR & Alternate: SAF199 |  |
| Additional Jlab Training (list): |  |

| Step | Description | Hazard | Control Measures |
| --- | --- | --- | --- |
| 1 | Pre-Job Briefing (PJB) | None | Conduct a PJB with the TR/CM before the job begins. |
| 2 | Toolbox Talk | None | Before each work shift, conduct a toolbox talk with all involved employees. Discuss the task(s) to be performed for the work shift and any associated hazards. |

| Tracking/Submittal Number | Project | Company Name |  |
| --- | --- | --- | --- |
| Task Description |  | Competent Person / SSHR Assigned |  |
|  |  | Phone: |  |

| Task Specific Training and/or Certifications Required |  |  |  |
| --- | --- | --- | --- |
| Step(s) | Required Training/Certification | Step(s) | Required Training/Certification |

| Tracking/Submittal Number | Project | Company Name |  |
| --- | --- | --- | --- |
| Task Description |  | Competent Person / SSHR Assigned |  |
|  |  | Phone: |  |

Signature Sheet

| I have been briefed on the hazards and control measures that are in place for this task. I also acknowledge that if conditions change or new hazards are discovered, I shall stop, place the work in a safe condition, and contact the Competent Person / SSHR. |  |  |
| --- | --- | --- |
| Print Name | Signature | Date |